COMMISSIONING RECORDS

PAC. performance test report  (Water Cooled) FORM TACF10-1

| Project: |  | Tag No.: |  |
| --- | --- | --- | --- |
| Job No.: |  | Apartment No.: |  |
| Date: |  | Tested By: |  |

# PAC

| Make | Model | Serial Number |
| --- | --- | --- |

Notes:

- Measure all parameters with all plant operating under normal conditions. E.g.: All the air path to be tight & all access covers to be closed..

- Measure all parameters after no less than 15 mins of solid operation. This is to give the PAC time to reach max operating performance

# AIR TEMPERATURE

| Where | Wet bulb | Dry Bulb | % RH | Comments |
| --- | --- | --- | --- | --- |
| Return Air | C | C |  |  |
| Supply Air | C | C |  |  |
| Note: Measure RA as close to the PAC evaporator as possible & with all access covers closed. Similarly the SA should be measured at the nearest outlet. |  |  |  |  |

# AIRFLOW

| What | Design | Measured | Fan Speed L/M/H ? | Comments |
| --- | --- | --- | --- | --- |
| Supply Air | L/s | L/s |  |  |

# WATER PERFORMANCE

| What | Leaving | Entering | Comments |
| --- | --- | --- | --- |
| Water Flow | L/s | NA |  |
| Water Temp | C | C |  |
| Note: On Temperzone water cooled PACs design differential water temp is approx 5.5 C. |  |  |  |

# COMPRESSOR PERFORMANCE

| Running Amps | Comments |
| --- | --- |
| A |  |

# PAC CAPACITY

|  | Design | Actual | Comments |
| --- | --- | --- | --- |
| Latent Capacity | kW | kW |  |
| Sensible Capacity | KW | KW |  |
| Total Capacity | KW | kW |  |

Comments: